Assignment module 2 : Installation and Maintenance of Hardware and Its

Section 1: Multiple Choice

1. Which of the following precautions should be taken before working on

computer hardware?

a) Ensure the computer is plugged in to prevent electrostatic discharge.

b) Wear an anti-static wrist strap to prevent damage from electrostatic

discharge.

c) Work on carpeted surfaces to prevent slipping.

d) Use magnetic tools to handle components more easily.

Answer :- b) Wear an anti-static wrist strap to prevent damage from electrostatic discharge.

2. What is the purpose of thermal paste during CPU installation?

a) To insulate the CPU from heat.

b) To provide mechanical support for the CPU.

c) To improve thermal conductivity between the CPU and the heat sink.

d) To prevent the CPU from overheating.

Answer :- c) To improve thermal conductivity between the CPU and the heat sink.

3. Which tool is used to measure the output voltage of a power supply

unit (PSU)?

a) Multimeter

b) Screwdriver

c) Pliers

d) Hex key

Answer :- a) Multimeter

4. Which component is responsible for storing BIOS settings, such as date

and time, even when the computer is powered off?

a) CMOS battery

b) CPU

c) RAM

d) Hard drive

Answer :- a) CMOS battery

Section 2: True or False

5. When installing a new hard drive, it is essential to format it before use.  True

6.  A POST (Power-On Self-Test) error indicates a problem with the CPU.  False

7.  It is safe to remove a USB flash drive from a computer without ejecting it first.  False

Section 3: Short Answer

8. Describe the steps involved in installing a new graphics card in a desktop computer.

Answer:-

### Install a New Graphics Card

#### 1. Open the Computer Case

- Remove the screws on the side panel of your desktop case.

- Slide the panel off to expose the motherboard and internal components.

#### 2. Locate the PCI Express (PCIe x16) Slot

- Find the long PCIe x16 slot on the motherboard — this is where the graphics card will go.

- If an old graphics card is installed, remove it by unscrewing and gently pulling it out.

#### 3. Remove the Expansion Slot Cover

- At the back of the case, remove the metal slot covers aligned with the PCIe slot.

- These covers allow the graphics card ports (HDMI, DisplayPort, etc.) to be accessible from outside.

#### 4. Insert the New Graphics Card

- Hold the graphics card by its edges.

- Line it up with the PCIe slot.

- Press it down firmly and evenly until it clicks into place.

- Secure it to the case with a screw.

#### 5. Connect Power Cables

- Many modern GPUs require additional power connectors (6-pin or 8-pin).

- Connect the appropriate PCIe power cables from your power supply unit (PSU).

#### 6. Close the Case and Reconnect Cables

- Reattach the side panel and screw it back in place.

- Plug in your power cable, monitor cable (to the GPU), keyboard, and mouse.

#### 7. Power On and Install Drivers

- Turn on your computer.

- Install the latest GPU drivers

#### 8. Test the Graphics Card

- Check display output.

- Verify driver installation in Device Manager (Windows).

- Run a simple benchmark or game to ensure it’s working properly.

Sortly :-  Power off → Open case → Insert GPU in PCIe slot → Connect power → Close case → Install drivers → Test

9. What is RAID, and what are some common RAID configurations ?

Answer :-

RAID (Redundant Array of Independent Disks) is a data storage technology that combines multiple physical hard drives into a single logical unit to improve performance, data redundancy, or both.

It is commonly used in servers, workstations, and systems where speed and reliability are important.

### ⚙️ Main Purposes of RAID

- Data Redundancy → Protects data if a drive fails.

- Performance Improvement → Increases read/write speed by distributing data across drives.

### Common RAID Configurations

#### 1. RAID 0 – Striping

- Purpose: Performance

- How it works: Data is split (striped) across two or more drives.

- Pros: Very fast read/write speeds.

- Cons: No redundancy — if one drive fails, all data is lost.

- Minimum drives: 2

🧠 Example:
Data A and B are split between two drives:

- Drive 1 → A1, B1

- Drive 2 → A2, B2

#### 2. RAID 1 – Mirroring

- Purpose: Redundancy

- How it works: Data is duplicated (mirrored) on two drives.

- Pros: High data protection — if one drive fails, data remains safe.

- Cons: Storage capacity is cut in half (because both drives store the same data).

- Minimum drives: 2

🧠 Example:

- Drive 1 → A, B, C

- Drive 2 → A, B, C

#### 3. RAID 5 – Striping with Parity

- Purpose: Balance between speed and redundancy

- How it works: Data and parity information are distributed across all drives.

- Pros: Can tolerate one drive failure without data loss.

- Cons: Slightly slower write speed due to parity calculation.

- Minimum drives: 3

🧠 Example:

- Drive 1 → Data A

- Drive 2 → Data B

- Drive 3 → Parity (A ⊕ B)

#### 4. RAID 10 (RAID 1 + 0) – Mirrored Striping

- Purpose: High performance and redundancy

- How it works: Combines RAID 0 (speed) and RAID 1 (mirroring).

- Pros: Fast and fault-tolerant.

- Cons: Expensive (requires more drives).

- Minimum drives: 4

Section 4: Practical Application

10. Demonstrate how to replace a CPU fan in a desktop computer.

Answer :-

### 1. Power Off and Unplug the Computer

- Shut down your PC.

- Unplug the power cable and all connected peripherals.

- Press the power button once to discharge any leftover electricity.

### 2. Open the Computer Case

- Remove the side panel of your desktop tower using a screwdriver.

- Place the case on a clean, flat surface.

### 3. Locate the CPU Fan and Heatsink

- The CPU fan is mounted on top of a metal heatsink, which sits directly above the CPU socket on the motherboard.

- Note how the fan’s power cable is connected to the CPU_FAN header on the motherboard.

### 4. Disconnect the Old CPU Fan

- Gently unplug the fan cable from the motherboard’s CPU_FAN connector.

- Unscrew or unclip the fan/heatsink assembly from the motherboard bracket.

  - Intel: push-pin clips.

  - AMD: metal retention brackets or screws.

- Carefully lift the fan and heatsink off the CPU.

### 5. Clean the CPU Surface

- Use isopropyl alcohol (90%+) and a soft cloth or paper towel to remove old thermal paste from the top of the CPU and the bottom of the heatsink.

- Ensure both surfaces are clean and dry before proceeding.

### 6. Apply New Thermal Paste

- Apply a pea-sized drop of thermal paste in the center of the CPU.

- This helps improve heat transfer between the CPU and heatsink.

### 7. Install the New CPU Fan

- Place the new fan and heatsink carefully on top of the CPU, aligning it with the mounting holes or bracket.

- Secure it using the provided screws or clips (tighten diagonally for even pressure).

- Plug the fan power connector into the CPU_FAN header on the motherboard.

### 8. Close the Case and Reconnect Power

- Replace the side panel and secure it.

- Reconnect the power cable, monitor, and peripherals.

### 9. Power On and Test

- Turn on the PC.

- Enter the BIOS/UEFI (press DEL or F2) to check that the CPU fan is spinning and recognized.

- Monitor the CPU temperature to ensure cooling is working properly.

✅ Note : -

Power off → Open case → Disconnect old fan → Clean → Apply thermal paste → Install new fan → Connect cable → Test

Section 5: Essay

11. Discuss the importance of regular maintenance for computer hardware and provide examples of maintenance tasks.

Answer :-

Regular maintenance of computer hardware is essential to ensure that a computer runs efficiently, lasts longer, and avoids unexpected breakdowns. It helps maintain performance, reliability, and safety of the system.

#### a) Improves Performance

- Dust, outdated drivers, or fragmented storage can slow down the system.

- Maintenance keeps the system running fast and responsive.

#### b) Prevents Hardware Damage

- Dust and overheating can damage internal parts like CPU, GPU, and power supply.

- Cleaning helps prevent overheating and component failure.

#### c) Increases Lifespan

- Regular care ensures longer hardware life and delays costly replacements.

#### d) Enhances System Stability

- Reduces crashes, freezes, or data loss due to hardware wear or failure.

#### e) Ensures Data Safety

- Regular backups and inspections help protect important files from hardware-related failures.

Example :- Cleaning , Checking Connection , Thermal Maintanance , Hard Drive Care , Power Supply Inspection , Peripheral Cleaning , Software Update(Related To Hardware) , Backup Important Data.